RSC Box Calculator

[Custom Made - RSC 1024 (1).xlsx]

[RSC Custom Made Calculation.pdf]

# Questions:

Are there restrictions when inputing LxWxH for AF/BF/DW

Types of paper:

Single wall - contains 3 layers of paper

A Flute 5mm thickness  - corrugation pattern

B Flute 3mm thickness - corrugation pattern

Double Wall 8 mm thickness - contains 5 layers of paper

DW = A Flute + B Flute

Blue join - 32 mm

External dimension refers to the folded up size of the box. L x W x H

Ref open size refers to the area of the box when it is unfolded

Quality of paper M/T/K/W

T - test liner

Cheapest recycled

K - virgin paper

Aligned fibres, stronger

W - white paper

White colour paper, expensive

M - Medium paper

Usually used for corrugation as the middle layer

Density of paper will affect the quality.

Minimum liner meter - MOQ

How to understand the paper quality notation?

Example: AF T150/M150/T150 - this is the name of the type of paper

T150 - the 150 is paper density of 150g/inch2

Example: DW T150/M120/M150/M180/T150

M = 150/120/180

What is the filtering/selection logic to select the correct sheet board from the reference table based on user input in Page 4?

What is the meaning of AF/BF/DW in paper weight calculation?

DW =  Double Wall?

SW = Single wall?

What are all possibilities?

For each sheet borad there seems to be outer, medium and inner quality quantities, what do they mean?

Meaning of Each quality M/T symbols?

What is T/ K/ W ? And how will it affect the board quality calculation

When will Ref Open SIze(mm) is used?

Ref open is for display purpose, no calculation needed, Ref open size is for reference when box is opened up to a full sheet (will reword it later)

Glue joint size for AF = 35mm, BF = 32mm, DW = 38mm.

No of Ups - number of boxes can be cut from a single pass of the sheet through the machine

Varies from supplier, machine to machine

Paper roll width calculation

Ref A - + 2 And round up

Ref B +4 and round up

Ref C - S/W + 20mm DW +24mm

Max roll width = 2.5m

Min roll width = ?

Roll width round up,

1100mm-1750mm to nearest 50mm

1800mm-2200mm to nearest 100mm

Printing cost: Flexo/printing

Ink cost = ?

Glue cost= ?

# Notes:

Estimated effort:

FE: 4 days

BE: 5 days

Test : 1 day